Johnnys Drilling and Pump Service

16973 Chuchupate Trl.

Frazier Park CA. 93225

808 825 2594

Lic # 846366

To whom it may concern,

Flow test: 1 domestic well

Property Located at 1085 Dusty Trl., Frazier Park CA. 93225

Flow tests conducted Feb. 20th 2025

Well #1

Depth 160’-0”

Pump set at 151-0”

Casing Dia. 6 inch PVC casing

Galvanize 1 drop pipe.

Pump size is a 1/2 horse power 230 volt submersible pump.

Water static level 114’-0”

Pump discharge rate ave. is 5 gallons per min.

Well number one  or south well recovers at a rate .125 gpm.

Pumped well at a rate of 5 gallons per min. for 18 min. before the pump broke suction and the safety mechanisms shut down the well pump.

Thank you

John Aguilar