| Permit #: | BPS26-000258 |
| --- | --- |
| Address: | 1529 S MCKINLEY AVE |

- Obtain all necessary permits for repairs or replacements listed.

EXTERIOR:

- Remove trash and debris away from the property.

- Cut tall grass; trim trees; vegetation away from building.

- Repair damaged kitchen cabinets.

- Weather protection (paint) has deteriorated.

- Repair or replace dry-rot: rafter tails/ facia boards

- Repair or replace broken window(s): permits required for replacement.

- Repair damaged foundation vents.

- Repair damaged walls, ceiling, flooring: crack floor tile (cut hazard)/ hole in wall under kitchen sink

- Repair damaged exterior walls.

- Seal and/or weatherproof exposed wood.

- Repair holes in paved surfaces (unsafe): trip hazards.

- Obtain necessary approvals and permits for:
[ x ] Addition/alteration: Legalize or demolish enclosed patio cover/ Close opening from garage to bedroom
[ ] Remodel
[ ] New structure
[ ] Garage conversion
[ ] As-builts
[ x ] Electrical: Legalize meter panel upgrade
[ x ] Plumbing: Legalize water heater replacement
[ ] Mechanical
[ ] Other

- Provide 36in landing at exterior side door.

ELECTRICAL:

- Main meter service panel and/or sub-panel(s)

- Provide proper bonding of water and gas piping

- Provide 2 ground rods minimum 6ft apart. Permits required.

- Label ALL circuit breakers

- Correct unsafe/hazardous wiring: exposed exterior light fixture wiring.

- Install Ground Fault Circuit Interrupter (GFCI) protection if ANY work has been done with in the past three (3) years, or if GFCI receptacles are not working properly.
[ x ] Kitchen counter-top areas
[ x ] Bathroom sink and vanity area.
[ x ] Garage
[ x ] Exterior
[ x ] Label GFCI receptacle outlets

- Replace or install new light fixture cover(s)

- ANY electrical repairs and/or work MUST be do ne by a California licensed contractor or a qualified electrical journeyman.

WATER HEATER:

- Pressure re lief drain line MUST extend outside of the building or to an APPROVED location.

- Install gas line sediment trap.

- Ensure exhaust vent terminates to the exterior of the building and above roof line.

PLUMBING:

- Flexible p-trap is not permitted for use with kitchen and bathroom sinks. Must use smooth wall piping.

MECHANICAL:

- Heater needs cleaning.

MISCELLANEOUS:

- Repair or replace entry lock(s).

- Submit construction one (1) sets of drawings for unpermitted work.