PRELIMINARY INSPECTION REPORT

To notify you of the condition of the septic system and the repairs needed, recommended contractors contact info is included.

NAME:  Dora Maxey

PROPERTY ADDRESS: 23808 Leisure Dr., Twain Harte

DATE: July 28, 2026

SEPTIC TANK:  1000-gallon, 2-lid concrete septic tank; pumped on July 22, 2026. The tank was found to be in good working condition at the time of inspection.  The lids are 2’ deep in the driveway the dirt is extremely hard recommend risers.

SEWER LINE:  The sewer line was flush tested; the flush came in but the down spout needs repair/replacement and resealed prior to passing.

LEACH FIELD: The leach field was water tested and no runback in the tank, no perc up in the field on the inspection date.

The septic system does not pass currently. We recommend the following for the repairs:

Otto B Plumbing * Kade Barajas * 209-559-8890

CMR Plumbing and Drain * Chris Rangle * 209-768-9151

Dave’s Plumbing * 209-984-3498

When repaired send photos to our office 209-533-1950 for review.

Respectfully Submitted,

Nick Marlowe

Mother Lode Septic Inc.

Tuolumne County Permit #PT0001684

NAWT/COWA Wastewater Transporters Inspector Cert. ID #114631ITC; OM1; OM11 Operations & Maintenance 1 & 11. 38+ years of pump/inspect experience.