PRELIMINARY INSPECTION REPORT

To notify you of the condition of the septic system and the repairs needed, recommended contractors contact info is included.

NAME:  Mason Hill and Megan Dean

PROPERTY ADDRESS: 23090 N. Bald Mountain Rd., Sonora

DATE: December 1, 2025

Red Colored House Upper Primary Tank:

SEPTIC TANK: 1000-gallon, 4 lid, concrete septic tank. The septic tank was found to be leaking and will need to be replaced.

SEWER LINE: The sewer line was flush tested and free flowing on the inspection date.  Red colored house and red colored cabin both sewer lines come into the upper primary tank.  Both inlets have roots and need to be removed.

LEACH FIELD: The leach field was water tested and no runback in the tank, no perc up in the field on the inspection date.

Lower Green Colored House:

SEPTIC TANK: 500-gallon, 1 lid, steel septic tank. The tank is in degradation and falling apart it will need to be replaced.

SEWER LINE: The sewer line was flush tested and free flowing on the inspection date.

LEACH FIELD: The leach field was water tested and no runback in the tank, no perc up in the field on the inspection date.

We recommend one of the following for your repairs:

Dean Stone Construction 209-352-9809

Brendan Simunaci 209-532-8718

Richard Berchtold 209-770-0232

The septic system does not pass currently. When repaired/replaced notify us and send photos.

Respectfully Submitted,

Nick Marlowe

Mother Lode Septic, Inc.

Tuolumne County Permit #PT0001684

NAWT/COWA Wastewater Transporters Inspector Cert. ID #114631ITC; OM1; OM11 Operations & Maintenance 1 & 11. 38+ years of pump/inspect experience.